Analysis software

for

Flow cytometric data

version 3.1

17 July 1992

by D.Vaulot

CNRS et Université Pierre et Marie Curie

Station Biologique, BP 74

29682 Roscoff Cx,  France

e-mail: Vaulot@univ-rennes1.fr

Tel: (33) 98 29 23 23

Fax: (33) 98 29 23 24

CYTOPC is distributed free of charge 
under the agreement that users acknowledge its origin 
if the program is used for scientific research resulting in publication.

The author would welcome reprints of papers that make use of the program.  Users are also kindly requested to communicate bugs they find.

Table of contents

Foreword

Hardware Requirements

Required

Optional

Language

Revisions

Acknowledgments

I. Getting started (CYTOPC)

Software Installation

Printer setup

Plotter (see Appendix 7)

Pointing device

Ready to go ?

Menus

Prompted Fields

In trouble ?

II. Processing histogram  and list mode (C-HIST)

File formats

Starting the program

Main Menu

A - Config PC

B - Config DATA

C - Files

D - Graf

E - List (List files only)

G - Statistics

H - Go

I - Quit

Statistics menu

Printing the Screen

III. Conversion of files (C-CONVER)

Starting the program

Input

Output

Examples of output files

1 parameter

2 parameters (LOTUS)

List mode

Appendix 1 - C-hist menu structure

Appendix 2 - Screen colors

Appendix 3 - Lin/log calibration of adcs

Appendix 4 - File transfer from MDADS or Profile to PC-compatible (Easy2 format)

MDADS setup (Only for EPINET)

Sysset

Menu Sysset

Epinet

Menu EPINET

Propas

Appendix 5 - File storage

Appendix 6 - Software notes

Appendix 7 - Contour drawing using SYSTAT and SYGRAPH (2P 64x64)

Inside CYTPOC:

Inside SYSTAT (see MAP.CMD file)

# Foreword

CYTOPC is a set of programs designed to analyse flow cytometer data in one of the following forms : single parameter histograms, two-parameter cytograms and list mode.  It is primarily meant for analyzing data generated by Coulter instruments such as the EPICS 5 or the Profile and transfered to the IBM PC through the EASY2 software.  It can also analyse other distributions such as Coulter Counter volume distributions generated by the C1000 Channel Analyzer.

CYTOPC has been written by D. Vaulot from CNRS, Station Biologique, 29680 Roscoff FRANCE, with help from Jeff Dusenberry (MIT).

This programm should cited in scientific papers as:

Vaulot, D. 1989. CYTOPC: Processing software for flow cytometric data. 
Signal and Noise 2:8.

## Hardware Requirements

### Required

- IBM-PC or compatible with hard disk.

- 400 K of free memory.

- Graphics screen adapter. One of the following:

. Hercules

. VGA

. EGA 256 K (16 colors)

. AT&T 6300 or Olivetti M24

. Genoa EGA

See Appendix 2 for color mapping.

### Optional

- Mathematical coprocessor 8087

- IBM compatible, HP PaintJet or LaserJet graphics printer

- Microsoft Mouse or compatible

- HP plotter (HP GL language).

Please contact D.Vaulot if your equipment is not supported to see what can be done.

## Language

CYTOPC is written in Turbo Pascal and compiled with version 6.0 (Borland, Scotts Valley CA).  Subroutine libraries from Turbo Halo (IMSI, Media Cybernetics, San Rafael CA USA) and from Turbo Professionnal (Turbo Power, Scotts Valley CA USA) have been used.

## Revisions

- 2.1 (August 1988)

- 2.3 (July 1988)

- 3.1 (Avril 1992)

## Acknowledgments

Help in implementing some of the functions of CYTOPC has been provided by Jeff Dusenberry (Woods Hole Oceanographic Institution).

# I. Getting started (CYTOPC)

## Software Installation

Put the CYTOPC disk(s) in drive A: and type.

C:

MD \CYTO

CD \CYTO

COPY A:*.*

These operations copy the following files on the hard disk.

CYTOPC   EXE

C-HIST   EXE

C-HIST   OVR

C-CONVER EXE

C-CYTOL  EXE

C-HIST   CFG

HALO     EXE

HALOKBDI COM

HALO106  FNT

HALOHERC DEV

HALOIBME DEV

HALOIBMV DEV

HALOINDA DEV

HALOLJTP PRN

HALOEPSN PRN

HALOPJET PRN

## Printer setup

The CYTOPC software makes use of a printer to record statistical results and to provide screen hard copies.  At present the following printers are supported:

- IBM Graphics Printer or EPSON compatible.

- HP PaintJet (Color).

- HP Laser Jet.

- Other printers can be used if they emulate if they emulate of the preceding ones.

The printer must to be attached to the current printer port (LPT1 or COM1) and turned on line.  Use the DOS command MODE to switch between serial and parallel ports.  A printer spooler might be handy to speed up operations (e.g. screen dumps).

## Plotter (see Appendix 7)

In some case you might want to have a higher quality output for your graphs.  You have three possibilities.

1 - You can use C-CONVER to convert data files from Easy to ASCII form and then use a program such as SIGMAPLOT for 1 parameter distribution or SURFER for 2 parameter distributions.

2 - When the Plotter option is turned On, an ASCII file (extension .CON) containing the coordinates of each contour is generated.  This file can be plotted using a graphing software such as SIGMAPLOT or SYGRAPH.

## Pointing device

CYTOPC supports two types of locators (pointing devices):

- Microsoft compatible Mouse.  The proper driver (e.g. MOUSE.COM, see the documentation of your mouse) must be loaded before running CYTOPC.

- The keyboard can be used to simulate the mouse.  In this case you must run the program HALOKBDI which loads the corresponding driver before using programs which use a pointing devices (C-HIST).  The numeric keypad arrows simulate the mouse motion and the Left Shift key the left button of the mouse.  Esc key toggles between mouse simulation and normal keyboard input.

## Ready to go ?

When you have checked that everything is OK, turn on your printer then type:

CD\CYTO

HALOKBDI	{if you don't have a mouse}

HALO

CYTOPC

... and there you go.  From the main menu you can choose one of the programs which make up CYTOPC to process or to convert your data.  All programs are largely menu-driven and most options are self explanatory.

### Menus

Moving between the options is achieved with the Left, Right, Up and Down arrows depending on the menu direction. Selection is performed with the Return key. Esc key returns to previous level.

### Prompted Fields

Editing within a field can be done with the Left and Right arrows and the End and Del keys (WordStar like commands).

### In trouble ?

If you have carefully read the manual and set up the different hardware options right, everything should go smoothly.  However since nobody is perfect here are some common mistakes and problems.

From the main CYTOPC menu, nothing happens when you want to start the other programs.  Either you do not have on your disk the correct files i.e C-HIST,  C-CONVER and PIC or there is not enough memory to start these programs.  In the former case go through the installation steps again.  In the latter case, you should remove any memory resident programs such SIDEKICK you may have loaded before running CYTOPC.  If this does not make it yet, go into your CONFIG.SYS file and remove any fat driver.  If this is not enough yet, then run C-HIST directly from DOS.  Does not work yet?  Well go to the shop and by more memory to get to the full 640 K allowed by DOS.

Once in C-HIST, after selecting all files correctly and starting processing, the system suddenly hangs up and you have to power down the computer in order to get out:  You forgot to load HALO and therefore the graphics driver are not installed.  Note that the HALO driver can be unloaded by typing HALO -u

After selecting the files correctly, the program does not want to process them. This is most likley a memory problem: see above for potential solutions.

Some files work, some don't.  This might happen if you are processing files from different origin, such as ASCII and Easy files, at the same time.  Since settings are different between these files you cannot process them together.

# II. Processing histogram  and list mode (C-HIST)

Program C-HIST plots and does statistical analyses of 1 or 2 parameter histogram and list mode EASY files created with M86PAS, EPINET or PROPAS.  It can also process ASCII files imported from the C1000 Channel Analyzer (Coulter counter volume data).

## File formats

The following files types are supported:

* ASCII : 1P histogram, one channel value per line (includes Coulter Counter files).

* Binary : 1 and 2 P histograms, each channel must be stored as 2 bytes.

* Coulter Easy: EPICS or Profile (FCS 1.0):

- 1P 256 channels

- 2P 64 by 64 channels

- List mode, 7 ADCs maximum, each ADC value stored as a byte (1..256).

* FACScan list files (FCS 1.0).  These files can be transferred either through disk conversion or thru FACSnet (Ethernet link).  Disk-converted or FACSnet-transferred files are analysed by a different versions of CYTOPC.  Versions before 3.1 analyse disk-converted files; version 3.1 analyses FACSnet files.  If you do not have the version suited to your needs, write to obtain it.

## Starting the program

C-HIST can be started from the main menu within CYTOPC or by typing directly C-HIST at the DOS prompt (see Chapter I).  Note that the Halo Graphics driver (HALO) must be loaded (see Chapter I).  If you do not have a Microsoft compatible mouse, you must also load the keyboard driver (HALOKBDI).  These files must only be loaded once: if you exit C-HIST and restart it again without rebooting do not reload them.

## Main Menu

The main menu (see structure in Appendix 1) permits access to all the settings of C-HIST.  A short help for each item is provided on line 25.

### A - Config PC

This option permits to change the PC hardware setup: screen, printer and pointer devices.  This must be done the first time you run C-HIST on a new system.  When all the settings have been changed to your liking, you can save them in a configuration file which has the extension .CFG.  Besides your harware configuration, most menu selections will be saved including ADCs settings, directory and file names, screen configuration as well as statistical windows.  Previous settings can be restored by selecting one of the available configuration files.

Screen  type : EGA/VGA/Hercules/Genoa/ATT

Printer type : HPPaint corresponds to HP PaintJet color printer.  HPLaser corresponds to the HP LaserJet.  IBM corresponds to any EPSON/IBM compatible printer.  The printer must be attached to the parallel port LPT1.

Plotter : Toggle ON if you want to create files for delayed plotting of contours (for 2 parameter distributions only) Files with extension *.CON contain an ASCII list of all contour points.  For each contour, the first line contain the height of the contour (h) and the number of points forming the contour (n).  The following n lines contain the X and Y coordinates of the contour points.  This format is compatible with the MAP format of SYGRAPH but can be used as input for any graphing software (see Appendix 8).

Mouse type :  Mouse corresponds to any Microsoft-compatible mouse.  Summa corresponds to Summaplus tablet.  Keybd corresponds to the keyboard emulation of the mouse.  In all cases the corresponding driver must be loaded prior to running CYTOPC (see I. GETTING STARTED).

Read Config.  You are provided with a list of configuration files (with extension *.CFG, the default file loaded first is called C-HIST.CFG).  Move with the keypad arrows to the desired location and type RET to confirm your choice.

Save Config. You are provided with the name of the current configuration file.  If you want to save the current configuration under a different name then modify it.  If you type ESC and you will be offered a choice of the available configuration files.

### B - Config DATA

This option permits to change the set up and calibration factors either for the MDADS or the Coulter

Important remark. In list mode it is assumed that there is no gap in the ADCs which are recorded.  For example if on the MDADS you have the following configuration:

ADC 1,3,4,5,6	on

ADC 2		off

then CYTOPC will assume that:

ADC 1,2,3,4,5	on

ADC 6		off

therefore:

ADC 1 of MDADS corresponds to ADC 1 of CYTOPC

ADC 3 of MDADS corresponds to ADC 2 of CYTOPC

ADC 4 of MDADS corresponds to ADC 3 of CYTOPC

ADC 5 of MDADS corresponds to ADC 4 of CYTOPC

ADC 6 of MDADS corresponds to ADC 5 of CYTOPC

MDADS ADC scales, calibration factors and names (next 2 items) must be set accordingly.

MDADS ADC Scale : For each ADC you can specify whether the input signal is converted on a linear, a log or a polynomial scale.

- If the input signal is on a linear scale you are prompted for the amplification you used.  All results are then converted to an amplification of 1.

- If the input signal is on a log scale you are prompted for 2 numbers A and B which permit to go from the log scale to the linear scale.  The conversion formula is as follows:

XLin = A*10B*XLog/Total Number of Channels

The converted XLin corresponds to the linear signal recorded with the amplification set to 1 on 1 to 256 channel scale.  For example A = 1 and B = 3 corresponds to a 3 decade logarithmic scale.  A and B must be carefully determined for each of your amplifiers (see Appendix 3).

- If the input signal is on a polynomial scale you are prompted for 6 numbers which are the coefficients of a 5th degree polynomial such that:

XLin = A0 + A1*Xpoly + A2*Xpoly2+.....

MDADS ADC Names : For each ADC you can enter the corresponding parameter name (no more than 6 characters).  Names transfered thru the EPINET program are not considered.

Coulter Set up : When you are processing Coulter volume distributions, enter here the settings used to record the distributions.

Coulter Calibration : When you are processing Coulter volume distributions, enter here the settings used to perform the calibration as well the actual volume and mean channel of the beads used.

Channels : For files which do not contain any information about the number of channels (e.g. binary files), enter here the number of channels (e.g. 64 or 256).  This information is not needed for Easy files.

Parameters : For files which do not contain any information about the number of parameters (e.g. binary files), enter here the number of parameters (i.e. 1 or 2).  This information is not needed for Easy files.

### C - Files

Change the selection mode and type of the files to be processed.

Data Path : Valid MS-DOS path name where the data files reside (e.g. A: or A:\ or \EASY\DATA).

Data File Mask : Valid MS-DOS file name or file mask (e.g. DATA0001 or DATA*. or DATA*.LIS or DATA00??. etc..).  All the files corresponding to the mask will be plotted and analyzed.  These files must be of the Origin and Storage types defined below (see two last items).

First Data File : First file to be considered for processing (e.g. DATA0015).  If one answers by * , then all files are processed.  Warning :  data file order is the order in which files are listed in the directory and not necessarily the numerical order (e.g. DATA0005 may be located before DATA0001 if it has been written to the disk earlier).

Last Data File : Last file to be considered for processing (e.g. DATA0050).  If one answers by * , then all files starting from the first data file are processed.

Data File Increment : This parameter permits to process every nth data file.  For example if 5 histograms have been recorded for each sample and one wants only to process the first histogram (e.g FALS), then one would set this parameter to 5.

List/Select Files : Gives a screen listing of the selected files marked by a star.  By moving to a file and typing RET one can select/deselect a given file. F3 select all files while F4 deselect all files.  When satisfied with your selection press ESC.

Content of files : Permits to list the content (Sample Name, Sample Description etc...) of the selected files if they are in FCS 1.0 format.  The information can be echoed to the printer.

Origin (IMPORTANT) : Tells from which machine the data are coming from.  Coulter corresponds to 100 channels ChannelAnalyzer distributions obtained from the Coulter counter.  MDADS corresponds to Coulter EPICS and Profile data.  FACScan corresponds to Becton Dickinson format.  ImageHistogram and ImageProfile correspond to MATROX Image Analysis output.

Storage (IMPORTANT) : Tells how the data are stored.  For Coulter Channel Analyzer data choose ASCII.  For Coulter MDADS and FACScan choose Easy.  If you choose Binary then you must provide the number of Parameters and the number of Channels (see 2 - Config Data).  If you choose the wrong file type the program will not be able to read the data.

### D - Graf

Change some of the graf settings especially for 2P cytograms.

Horizontal : # of plots in the X direction for plotting.  Use typical numbers between 1 and 4.

Vertical: # of plots in the Y direction for plotting.  Use typical numbers between 1 and 4.

Type : Contour or Scatter (Cytogram only). Toggle to have contour lines displayed for 2 parameters cytograms.  This takes significantly longer than the scatterplot.

Smoothing : (Cytogram contour plots only).   The method is the count-dependent smoothing of Schuette et al. (Cytometry, 1986).  For linear parameters, the CV of the smoothing kernel is held fixed while for log parameters it is the SD which is held fixed and equal to the CV for smoothing (see below) multiplied by the total number of channels.  It requires two parameters:

Number of smoothing points : between 0 (no smoothing) and 4.  A kernel of 2N+1 by 2N+1 points around each point is used for smoothing.  Since processing time is proportionnal to N2 do not take this number too big.

CV for smoothing :  by increasing the CV one increases the smoothing effectiveness.  Ideally it should be equal to the CV due to the measurement error e.g. the CV obtained for very good beads.

Levels : (Two parameter cytograms only) Change the number of levels (maximum = 9) as well as the height, color (see Appendix 2) and dot size (for scatter plots) of each level.  In contour mode the first level is represented by dots.  Set color to 0 to prevent dot display.

Page Feed :  Toggle ON if you want a form feed to be generated automatically after each scrren has been printed.

Print Auto : Automatic Mode for Screen Harcopy. Toggle ON if you want the screen to be automatically copied to the printer when all plots on a page have been drawn.

Print Zoom : Changes the size of the screen printout.  Typical values are 0.5 (to have a small printout), 1, or 2 (for big printout).

### E - List (List files only)

In list mode, each cell is defined by the value recorded for each ADC (on a 1 to 256 scale).  For each list mode file, C-HIST allows you to generate up to 6 two parameters plots (Plots per File).  For each plot you must select which ADC will correspond to the X and Y parameters (ADC Selection).  If no statistical window is active, then all cells are plotted in the same color (White by default).

In order to identify subpopulations you must interactively define gates called by CYTOPC statistics windows (see below Statistics Menu).  Statistics windows are hypercubes/hyperbitmaps defined in the space of the ADC, e.g. a five-dimension space if 5 ADCs were recorded, by a combination of rectangular windows or polygonal bitmaps on two parameter cytograms.  To each statistical window corresponds a given color and cells which will fall inside the window are colored accordingly (Colors for windows).  When all the predefined plots have been achieved the statistics inside each active window are computed.

In list mode, cell values are stored in memory if there is enough space or directly read from the disk if the file is too big to hold in memory.  On a PC XT, computing statistics in 4 windows on a list file about 100 Kbytes long will take about 3 minutes if the list file is held in memory and 15 minutes if it is hold on disk.  Therefore it is advisable to remove from memory any unused software before running C-HIST for list mode data and to limit list files to 65000 cells.

Cells per Plot : To save time only a limited number of cells are plotted on each graph.  Usually from 1000 to 5000 cells are enough to be able to accurately define windows.  The statistics however are computed on the total numbers of cells held in the file.

Plots per File : (max = 6) Total number of plots to be generated for each list file.

ADC selection : Permits to choose which CYTOPC ADCs are chosen as X and Y parameters for each plot.  These are the CYTOPC ADCs and not the original ADCs  (see above b - Config data).

Colors for windows : Permits to change the colors for each of the statistical windows (max = 9) and for the cells which do not fall in any of the window.  If 0 is chosen for the latter cells, they are not plotted at all.

Window List Modes : If a given window is turned ON then the program will output an ASCII list mode file (extension .W*) containing only the cells falling into this window.

### G - Statistics

Windows are separated into bead and cell windows.  Bead windows contain reference particles while cell windows contain particles of interest for the analysis.

Print : Statistics Output to printer.  Toggle ON if you want the results of the statiscal analysis to be sent to the printer.

Report : If Brief is chosen, statistics are printed in a compressed way with one window per line with parameters arranged in columns.  Only number of cells, mean and CV are provided.  If Long is chosen the whole statistical information is output including window boundaries, mean, mode and CV.

File : Statistics output to file. Toggle ON if you want the results of the statistical analysis to be sent to files.  You are then prompted for the Name of the Files where the statistics will be stored.  You do not need to provide an extension.  Two files will be created with extensions *.STA and *.NOR.  These files will reside in the same directory than the data files and can be loaded into Lotus 123 or Excel using the File Import facility.  If files with the same name already exist, the new statistics will be appended to the existing files.

- The first file (*.STA) contains raw statistical data.  Each line corresponds to one parameter and one window and provides the number of cells in this window, the mean, the mode and the coefficient of variation of the parameter.

- The second file (*.NOR) contains normalized statistical data in compacted format.  For bead windows only the number of particles is given.  For cell windows, the file provides the number of particles per unit volume (mL), as well as the mean of each parameter normalized by the same parameter measured on a selected bead window.

File separator :  Choose Space if you want to load the statistical output files into Lotus and Tab for Excel.

Auto Mode : Automatic Mode for Statistics.  Toggle ON if you do not want to interactively define statistics windows (see below).  Automatic processing can be stopped by depressing any key: in this case one goes back to the manual mode without automatic hardcopy.

Reset all windows : Sometimes it is wise to reset all windows to their default value (1..256), especially when doing list mode processing with new files.

Bead windows:  For each ADC you may specificy a window (bead window) which will be used as a reference for this ADC in file *.NOR.  For example if you have 1 micron non-fluorescing beads falling in window 1 and 10 microns fluorescing beads falling in window 2 you may normalize FALS and LS90 (e.g. ADC 1 and 2) to window 1 and Green and Red fluorescence (e.g ADC 3 and 4) to window 2.  For a given ADC if you choose winodow 0, ADC statistics will not be normalized.

Cell windows :  First give the total number of windows which contain cells and then give the reference number of each the windows.  For example if windows 1 and 2 are bead windows, you can use windows 3, 4 and 5 for three different cell populations.

Sample volume : When running a fixed volume, enter this volume here (e.g. in mL).  The concentration of particles in each cell window will be rapported to volume unit in the .NOR file.

### H - Go

Begin to plot and process the data.  See paragraph Statistics Menu.

### I - Quit

Exit the program (make sure you have saved your configuration).

## Statistics menu

After each file has been plotted a certain number of options are available for statistical analysis. Briefly it is possible to define a maximum of 9 windows in which the following statistics are computed  :

- Mode

- Mean

- CV (in %)

Statistical results are computed on a linear scale (after eventual tranformation from the initial logarithmic scale: see above Calibration item in Main Menu).  For MDADS data the last channel is adjusted to 256 irrespective of the initial scale (i.e. 64 or 256).  For Coulter data the channel is converted to volume.

Rectangular windows and bitmaps are defined between 1 and the maximum number of channels (100 for Coulter data, 256 for 1 parameter and list modes MDADS histograms and  64 for 2 parameter MDADS histograms).  After resetting all windows (Main Menu g - Statistics) or in the absence of a configuration file, all windows are inactive and defined from channels 1 to 256.   On a given cytogram, a statistics window can be defined by either a rectangular window (W) or a polygonal bitmap (B). Bitmap vertexes are limited to 10.  The number of bitmaps is limited to 6.  Therefore, use only bitmaps when absolutely necessary.  Most of the time, rectangular windows are sufficient.

To change window boundaries use the W or B commands.  Rectangular windows and polygonal bitmaps are mutually exclusive: when you press W and give the number of a statistical window for which a bitmap was already defined, the bitmap will be erased and conversely.  When a window has been activated it remains in effect for all the subsequent files even after going back to the main menu.  To activate a previously defined window, use the A command.  To inactivate a window use the I command.  Histograms and Cytograms of  a window population can be stored with the  P command.

W : 	Change the limits of a rectangualr window. The user is prompted for the Window number  (between 1 and 9) and then for the limits of the window.  If a pointer device is present a cursor appears on the screen and the user must first click with the left button (Left Shift with the keyboard emulator) for the left bottom corner and then for the right top corner of the window.

B : 	(Only for 2P-cytograms) Change the limit of a bitmap. The user is prompted for the Window number  (between 1 and 9) and then for the limits of the window.  Click on the left mouse button to define a vertex.  Press on the right mouse button to close the bitmap.

A : 	Activate a window so that statistics will be computed in this window. The user is prompted for the Window number  (between 1 and 9)

I : 	Inactivate a window. The user is prompted for the Window number  (between 1 and 9)

P : 	(Only for cytograms and list modes) Store projections of the data points lying within a window into a file written in Easy format and which can thereofre reanalyzed with C-HIST. The user is prompted for the Window number  (between 1 and 9). If the data file is called DATA0001 then the projections of window 1 onto ADC 2 is stored into file DATA0001.12 with the first number (1) standing for for window 1 and the second number (2) for the  ADC 2.  In list mode the window projection onto the current 2P-cytogram is also stored.  For example if you are currently viewing ADC 2 versus ADC 3, projecting will create a file called DATA0001.123 containing the projection of window 1 onto ADCs 2 and 3.

RET : Compute statistics of the active windows and go to next file.

U : 	Process the next files in automatic mode, i.e without the possibility of modifying the statistics windows etc... It is a good idea to have also the Print Auto option on.  This mode can be stopped at any time by depressing a key: control returns then to the operator.

E :	 Perform a printer page feed.

Q : Quit to return to the main menu without computing the statistics of the current file and skipping the remaining files .

C : (Only for 1 parameter histograms) Change the vertical scale (Ymax) of the current histogram.

In List mode two additionnal commands are available:

F : Replay the current file.  Regenerate all the plots of the current file, taking into account the latest definition of the windows.  This is useful once all the windows have been correctly defined.

G : Replay the current graf.  Regenerate the current plot of the current file, taking into account the latest definition of the windows.  This is useful to check whether windows have been correctly defined.

## Printing the Screen

Screen printing requires a graphics printer (see above).  At the end of a screen plot enter (manual mode only):

*  to activate screen printing

RET to continue processing files without printing the screen.

# III. Conversion of files (C-CONVER)

Program C-CONVER converts files from EASY format to ASCII format and filters EASY List files.  Pressing the Return key starts processing.  Pressing the Esc key aborts the program.

## Starting the program

You can start the file conversion program either direcitly from CYTOPC main menu or by typing at the DOS prompt:

C-CONVER

## Input

Data File Path : Valid MS-DOS path name where the data file to be converted resides (e.g. A: or A:\ or C:\EASY\DATA).  The output file will also resides in the same path.

Data File Mask : Valid MS-DOS file name or file mask (e.g. DATA0001 or DATA*. or DATA*.LIS or DATA00??. etc..).  All the files corresponding to the mask and falling between the first and the last file name (see below) will be converted.

First Data File : First file to be considered for processing (e.g. DATA0015).  If one answers by * , then all files are converted.  Warning :  data file order is the order in which files are listed in the directory and not necessarily the numerical order (e.g. DATA0005 may be located before DATA0001 if it has been written to the disk earlier).

Last Data File : Last file to be considered for processing (e.g. DATA0050).  If one answers by * , then all files are converted starting from the first data file.

Mode of storage of converted file : (Only for List mode)  List mode files can filtered (see below) and then stored either in Easy (answer 1) or ASCII (answer 3) format.

Usage of converted file : (Only for 2P cytograms)  Cytograms can be converted to a plain ASCII file (answer 1) or to a specially formatted file which can be then imported directly in the SURFER program for contouring and 3-D plots (answer 2).

Filtering : For 2P cytograms filtering consists in setting to zero all elevations which are below a critical threshold.  For list mode files, you must choose an ADC and then only the cells which will have a value for this ADC larger than the threshold will be written to the output file.

ADC used to threshold list files :  (Only for list files) For example if you want to output only the cells which have a signal on ADC3 falling beyond channel 10 you should enter 3 here.

Threshold :  For the previous example you should enter 10 here.

Fraction of cells ignored (only for list mode ) : This permits to discard a fixed fraction of the cells at the beginning of list file.  This is useful in some cases when some of the parameters are drifting at the beginning of a run (e.g. DNA).

## Output

Output File Name.  The main part of the output file name is the same than that of the input file.  The extension begins by an U for 1 parameter histograms, by a C for 2 parameters cytograms and by and L for list mode files.  This letter is followed by a number which is unique to the file.  Both input and output file names are sent to the printer.  For surfer type files the extension is .GRD for

Example:  if the input file name is DATA0001 or DATA0001.PRN and the file contains list mode data, the output file name will be DATA0001.L01 if no file with the same name exists and DATA0001.L02 if a file with extension L01 is already present.

Output File Content

| Type | Output Choice | Format |
| --- | --- | --- |
| 1 parameter (256) | ASCII | 1 channel per line |
| 2 parameters (64x64) | ASCII LOTUS | 1 channel per line |
| 2 parameters (64x64) | ASCII Surfer | 8 channels per line |
| List | ASCII | 1 cell per line |
| List | Easy | FCS 1.0 format |

## Examples of output files

### 1 parameter

0	<-- Number of cells in channel # 1

0	<-- Number of cells in channel # 2

0	...

1

2

4

.....

### 2 parameters (LOTUS)

1   1      0	ï Number of cells in channel # 1,1

1   2     10	ï  Number of cells in channel # 1,2

1   3     25

1   4     20

..........

### List mode

ADC1	 ADC2 	ADC3 	ADC4

253 	147   	4 	104	ï cell number 1

253 	133   	5 	104	ï cell number 2

253 1	65  	12 	150

..........

# Appendix 1 - C-hist menu structure

Config PC

Screen  type :  EGA/VGA/Hercules/ATT/Genoa

Printer type :  HPPaint/HPLaser/IBM

Plotter      :  OFF/ON

Mouse   type :  Mouse/Summa/Keybd

Save Config

Read Config

Config Data

MDADS ADCs  : 7

MDADS ADC Scale

ADC 1:Lin/Log/Poly

ADC 2:Lin/Log/Poly

.........

MDADS ADC Name

ADC 1:FALS

ADC 2:LS90

.........

Set up Coulter

Scale       : Log

Current     : 1.2

Attenuation : 256

Gain        : 1

Base Chan   : 5

Window width: 70

Dilution    : 1.0

Calib. Coulter

Scale       : Log

Current     : 1.2

Attenuation : 256

Gain        : 1

Base Chan   : 30

Window width: 70

Volume Ref. : 520.5

Channel Ref.: 32

Channels   :   64

Parameters :    2

Files

Path :  d:\eros

Mask : eros*.

First File : *

Last File  : eros0017

Increment :  2

List/Select Files

Content of Files

Origin  :  Coulter/MDADS/Image Profile/Image Histogram/FACScan

Storage :  Easy (FCS 1.0)/Binary/ASCII

Graf

Horizontal : 2

Vertical   : 2

Type :   Scatter/Contour

Levels

Number

Height

Color

Smoothing

Number of points : 2

Assumed CV : 0.05

Page  feed : OFF

Print auto : OFF

Print zoom : 0.05

List

Cells per Plot : 2000

Plots per File : 3

Plot type

ADC selection

Plot 1 - ADC for X axis : 1

ADC for Y axis : 3

Plot 2 - ADC for X axis : 1

ADC for Y axis : 4

......

Colors for windows

Cells out of all windows - Color (0 to ignore them) : 15

Cells in window 1 - Color : 3

......

Window List Modes

Window 1 : ONN/OFF

Window 2 : ONN/OFF

......

Statistics

Print  : OFF/ON

Report : Brief/Long

File   : OFF/ON

Auto mode : OFF/ON

Reset all windows

Bead windows

ADC 1 : Normalizing window (0 if not) : 1

ADC 2 : Normalizing window (0 if not) : 2

.......

Cell windows

Number of cell windows : 3

Cell window # 1 : 3

Cell window # 2 : 4

.......

Sample volume : 0.5

Go

Quit

# Appendix 2 - Screen colors

Correspondance between colors  and numbers for ega and vga displays:

black1 	= 0         backgound (no color)

blue1  	= 1         dark blue

blue2  	= 2

blue3  	= 3

blue4  	= 4         light blue

purple1  	= 5

rose1  	= 6

rose2  	= 7

rose3 	= 8

red1  	= 9

red2  	= 10

yellow1 	= 11

yellow2 	= 12

green1  	= 13

brown1 	= 14

white1 	= 15

# Appendix 3 - Lin/log calibration of adcs

The A and B coefficients are amplifier specific.  in order to compute them for your own system, follow these steps for each amplifier :

1 - 	Put linear signal on ADC1 (amplification set to 1).

2 - 	Put log signal on ADC2.

3 - 	Create a protocol where ADC1 vs ADC2 (64x64) is collected.

4 - 	Run some beads and vary the amplifier high voltage so that you get a continuous trace on the ADC1 vs ADC2 histogram.  This trace is a plot of the linear channel (X) vs the log (Y) channel.

5 - 	Since A coefficient is defined for 256 channels and B does not depend on the number of channels you should have (cytopc manual p.11) :

X = (A/4)*10(Y*B/64)

i.e.

log X = (B/64)*Y + log (A/4)

6 - 	Therefore, you can obtain both A and B by computing the values of several couples of (X,Y)  on the curve and making a linear regression between log X and Y.  typical values with an EPICS 541 are A = 0.3 and B = 3.0 for the PMT amplifier and A = 1 and B = 2.5 for the FALS amplifier.

+	This calibration could also be done by collecting the log and the linear signals in list mode and regressing the two signals.

# Appendix 4 - File transfer from MDADS or Profile to PC-compatible (Easy2 format)

This appendix outlines the main fuctions of the programs provided by coulter to transfer data from the epics mdads to pc-compatible computers (easy2 format) using the epinet (ethernet) connection (programs sysset and epinet).  translation between profile and easy2 formats (program propas) works along the same line.

## MDADS setup (Only for EPINET)

Boot the system up to the MDADS level (after setting the date).

## Sysset

This program enables to change the directory and name of the files transferred from the EPICS.  Choose options in the following order:

#### Menu Sysset

3 : Edit System File Name: Menu SYSNAME

1 : Data File pathway (e.g. \MYDATA\). Important: this path must have been created previously through the DOS command MD (e.g. MD \MYDATA).

2 : Data File Prefix : beginning of the DOS file name (no more than 4 letter e.g. DATA).

1 : Write System File SYST.2.

## Epinet

Transfer takes about 15 minutes per disk with a Compaq 386.  Use F3 to rotate among the menus:

#### Menu EPINET

0 : Select Source EPINET  (to list and select files on the MDADS).

Menu EPISRC

0 : Directory of MDADS files

2 : Low file name (see EASY2 manual)

3 : High file name

1 : Program Controls.

Menu EPICON

4 : Active device. Permit to choose which of F0 or F1 is used on the MDADS.

1 : SD : Hard disk

2 : F0

3 : F1

5 : Data Directory Name (HIST or LMODE) (to change from histogram to list modes).

2 : Select Start and destination files (to set the index of the first file)

Menu MODDIR

5 : Set the Start File Index.  Important: Always set it different from 0. You can set the first two digits to the disk number (e.g. 2201 will be the first file transferred from disk # 22).

3 : Transfer data (to start the actual file transfer).

## Propas

Transforms Profile files into Easy2 files.  Identical to EPINET.

# Appendix 5 - File storage

Data compression thru the PKARC program can save up to 90% of space on the disk for 1 and especially 2 parameter histograms.  For list file, compressing is usually not worth the trouble since it only saves 10% of disk space.  One can for example compress all the files from the same MDADS disk in a single ARC file.  Let's assume that we have transferred data from MDADS disk DATA # 22 to PC files DATA2201. to DATA2250. located in directory \MYDATA.  Then the following ARC commands can be used:

PKARC A \MYDATA\DATA22 \MYDATA\DATA22??.

Appends files : adds all the data files to the archive file DATA22.ARC

PKARC M \MYDATA\DATA22 \MYDATA\DATA22??.

Moves files : same as previous but deletes at the same time the initial files from the disk

PKXARC \MYDATA\DATA22 \MYDATA\DATA22??.

Xtract files : extracts the data files from the archive file DATA22.ARC

# Appendix 6 - Software notes

The program C-HIST is written in Turbo Pascal version 6.0.  It uses 2 commercial software libraries:

- HALO88 (module Turbo Pascal) which manages all the basic graphic interface.

- Turbo Professionnal (all modules which begin by TP...) which contains a lot of useful subroutines (e.g. for menu construction, string manipulation etc...).

The program is organized into the following modules:

- C-HIST.PAS is the main calling program, it contains only synthetic routines and update information.

- CTYPE.PAS contains all type, constant and variable declarations which are used in the other modules and are global to the program.  New declarations MUST be included in this module.

- CMENU.PAS manages the main menu.  It relies totally on the subroutines from Turbo Professionnal.

- CFILE.PAS manages most of file reading and writing.

- GRAFTYPE.PAS defines all the types and variables necessary to GRAF.

- GRAF.PAS manages all low level graphics routines.  It relies on the Turbo Halo software library.

- CPLOT.PAS manages all the higher level graphics.  It is device indeendant.

- CCONTUR.PAS contains the main contouring routine for 2 parameters files.

- CSTAT.PAS manages all the computationnal procedures which are specific to the statistical processing of histograms and cytograms as well as the statistical output.

# Appendix 7 - Contour drawing using SYSTAT and SYGRAPH (2P 64x64)

## Inside CYTPOC:

1.	Project list modes to 2P if necessary

2.	Set 2P plots to contour mode

3.	Define the contour levels (note the first level that appears as dots will not be plotted)

4.	Set Plotter ON (Menu CONFIG PC)

5.	Select the files to be plotted

6.	Go

7.	For each file MYDATA, a text file named MYDATA.CON and containing the coordinates of each contour will be created.

## Inside SYSTAT (see MAP.CMD file)

Two files are needed:

-	MYDATA.MAP: 	contains the contours

-	MYDATA.SYS:	contains information about the contour

1.	Convert the MYDATA.CON file to a MYDATA.MAP file using the following SYSTAT commands:

DATA

Save "MYDATA.map" / Double

Import "MYDATA.con" / Type = Map

2.	Create a MYDATA.SYS file using EDIT which will have the following structure

| MAPNUM | MINLAT | MAXLAT | MINLON | MAXLON | LABLAT | LABLON |
| --- | --- | --- | --- | --- | --- | --- |
| 2 | 0 | 64 | 0 | 64 |  |  |
| 3 | 0 | 64 | 0 | 64 |  |  |
| 4 | 0 | 64 | 0 | 64 |  |  |
| 5 | 0 | 64 | 0 | 64 |  |  |
| 6 | 0 | 64 | 0 | 64 |  |  |
| 7 | 0 | 64 | 0 | 64 |  |  |
| 8 | 0 | 64 | 0 | 64 |  |  |
| 10 | 0 | 64 | 0 | 64 |  |  |

-	The name of the variables must appear exactly as stated.

-	MAPNUM corresponds to the heights of the different levels.  For exemple if you have on three levels then this file must contain only three lines.

-	The other columns must not be changed

3.	Plot the contours using the following commands:

SYGRAPH

Mode Video=VGA2 / Direct

Mode Printer=HPL2 / LPT1

Mode Metafile=CGM1

output=*

begin

use myfile

note 'plot the contours'

map /xmin=0,xmax=64, ymin=0,

ymax=64,hei=10cm,wi=10cm

note 'plot the axis in log mode /modify if one is in linear mode'

plot minlon*minlat/xmin=1,xmax=1000, ymin=1,

ymax=1000, axes=4,  xtic=3,xlog=10,xpip=9,

ytic=3,ylog=10,ypip=9,  hei=10cm,wi=10cm,

xlab="DNA",ylab="Chlorophyll"

end